TC Evan 2012 SHELTER GUIDELINES AND STANDARDS

SHELTER CLUSTER TECHNICAL ADVICE

# 1. INTRODUCTION

These ‘Technical Guidelines and Standards’ have been drafted by the Shelter Cluster co-lead, the International Federation of the Red Cross and Red Crescent Societies (IFRC), in their role as technical adviser. The Technical Guidelines and Standards are in line with the strategies held in the Shelter Cluster Operational Framework and are recommended to the Fiji Shelter Cluster.

The aim of these Guidelines and Standards is to ensure appropriate and equitable support regardless of ideology, race, political orientation, social or cultural background, and to set technical standards for the shelter response. They consist of:

2. SHELTER TERMS AND DEFINITIONS	3

2.1 Shelter Options

2.2 Shelter Assistance

3. DESIGN PRINCIPLES	6

4. SHELTER RESPONSE OPTIONS AND TECHNICAL STANDARDS	7

4.1 Tarpaulins and plastic sheeting

4.2 Tents

4.3 Shelter Kits

4.4 Temporary/Transitional Shelters

4.5 Support to Host Families

4.6 Permanent Housing

5. CROSS CUTTIMG CONSIDERATIONS	11

5.1 Environment

5.2 Gender

5.3 Age

5.4 HIV/AIDS

5.5 Disabilities

5.6 Human Rights

5.7 Livestock

6. INTER CLUSTER CONSIDERATIONS – to be completed	11

6.1 CCCM

6.2 Protection

6.3 WASH

6.4 Early Recovery

7. IMPLEMENTATION METHODOLOGIES	11

7.1 Cash Grants & Voucher

7.2 Cash for Work

7.3 Participation

7.4 Owner Driven Approach

7.5 Technical guidance, supervision and monitoring

Contact Information:

Cluster Lead:

Ministry of Local Government, Urban Development, Housing & Environment

FA House, Gladstone Road, Suva, Fiji

Vula Shaw, Principal Administrative Officer, 9066177

https://www.sheltercluster.org/Asia/Pacific/TCEvan2012/Pages/default.aspx

https://www.sheltercluster.org

coord.fiji@sheltercluster.org

# 2. SHELTER TERMS AND DEFINITIONS

## 2.1 Shelter Options:

| Shelter strategic options matrix |  |  |  |  |
| --- | --- | --- | --- | --- |
| Target groups |  | Objective of intervention | Emergency activities up to 4wks | Recovery activities 2wk to 24mth |
|  | Displaced Households (HH) living in evacuation centres (EC), e.g. schools. | Support provided to HHs to return to their original homes (either undamaged, damaged or destroyed). | Tarpaulins, tents & Non Food Items (NFI) | Formal Sector (FS), Government of Fiji (GoF) assistance for repair or ‘Core House’ if they qualify under Rehab Policy (Annex A). Others are expected to recover by themselves, ‘Build Back Safer’ education Informal Sector (IS), ‘Transitional Shelter’, if identified under the IS ‘Detailed Damage Assessment’ (DDA) or during later assessments by the implementing agency. ‘Build Back Safer’ education, IS, Resettlement through GoF program |
|  | Displaced HHs living with host families, etc. | Support provided to HHs during their stay with host families and to return to their original homes (either undamaged, damaged or destroyed). | Tarpaulins, tents & NFI’s | FS, GoF assistance for repair or ‘Core House’ if they qualify under Rehab Policy (Annex A). Others are expected to recover by themselves. ‘Build Back Safer’ education IS, ‘Transitional Shelter’, if identified under IS DDA or later assessments, ‘Build Back Safer’ education, IS, Resettlement through GoF program FS and IS, tarpaulins, NFIs, CGI, hardware vouchers |
|  | Non-displaced HHs living in partially damaged houses. | Support provided to HHs to repair their partially damaged houses. | Tarpaulins & NFIs | FS, GoF assistance for repair if they qualify under Rehab Policy (Annex A). Others are expected to recover by themselves IS, Tarpaulins & NFIs, CGI, hardware vouchers |
|  | Non-displaced HHs living in significantly damaged housing. | Support provided to HHs to repair or rebuild their significantly damaged houses. | FS, tarpaulins, tents, tools, corrugated galvanised iron sheets (CGI) & NFIs IS, Tarpaulins, tents, tools, CGI &NFIs | FS, GoF assistance for repair or ‘Core House’ if they qualify under Rehab Policy (Annex A). Others are expected to recover by themselves. ‘Build Back Safer’ education IS, ‘Transitional Shelter’, if identified under IS DDA or later assessments, ‘Build Back Safer’ education, IS, Resettlement through GoF program FS and IS, tarpaulins, NFIs, CGI, hardware vouchers |
|  | Non-displaced HHs living in makeshift shelters on the plots of their totally destroyed house. | Support provided to HHs to rebuild their destroyed houses or assist in relocation. | FS, tarpaulins, tents, tools, CGI & NFIs IS, Tarpaulins, tents, tools, CGI &NFIs | FS, GoF assistance for repair or ‘Core House’ if they qualify under Rehab Policy (Annex A). Others are expected to recover by themselves. ‘Build Back Safer’ education IS, ‘Transitional Shelter’, if identified under IS DDA or later assessments, ‘Build Back Safer’ education, IS and FS, Resettlement through GoF program - FS and IS, tarpaulins, NFIs, CGI, hardware vouchers |
| Notes: |  |  |  |  |
| - Government released ‘New Rehabilitation Housing Policy’ on 21 Dec 2012 that identifies qualification criteria for the FS to qualify for Government assistance (Annex A). - Republic of Fiji ‘The National Housing Policy’, Revised Edition 2012, contains information on the Fijian housing sector (https://www.sheltercluster.org/Asia/Pacific/TCEvan2012/Pages/default.aspx). The ‘National Housing Council’ has been established to complete a National Housing Act, but progress is currently hampered by the upcoming elections in 2014. - Where appropriate vouchers can be considered as a method of implementation – in coordination with appropriate support and monitoring mechanisms. These were used in 2012 flood response with all parties recognising the success but need for improvement. Refer to PCN, Rotary for feedback. - Support to renters should be as above, but rental assistance should be considered if an increase in rental cost occurs due to a shortage of rental properties. - All repairs and permanent structures must be provided in coordination with the appropriate level of technical training, monitoring and guidance. Transitional houses for the IS are to be engineer certified and members sized and signed off. HFHF is completing a construction manual. - All repairs and permanent structures should aim to comply with the appropriate recognised national standards (guidance is available from the National Housing Authority and Ministry of Health). - Maximum recommended time for tents and tarps as a place of main habitation is 3 months. |  |  |  |  |

## 2.2 Shelter Assistance: all shelter responses are targeted at family units

| Type of Shelter | Description |
| --- | --- |
| Buildings | Schools and public buildings used to provide emergency sheltering solution. |
| Host Families / Host Communities | Temporary accommodation provided by neighbours, relatives or communities within affected areas. |
| Tents | Portable shelters with a cover and a structure, used to provide emergency shelters. |
| Temporary / Transitional Shelter | Temporary/transitional shelter is shelter provided during the period between natural disaster and the achievement of a long-term shelter solution. |
| Partially Damaged House Shelter Repair Kit | Consists of construction material and/or tools, or in the form of voucher, to provide support for return to or at place of origin where houses were damaged (For both the Formal and Informal Sector). |
| Permanent ‘Core House’ | Built at place of origin or at relocation sites. |

# 3. DESIGN PRINCIPLES

| Indicators | Criteria: | Design Principles: |
| --- | --- | --- |
| Climate Suitability | - Ventilation | Design of the shelter to allow adequate ventilation to reduce internal temperatures. The design should allow for climate suitability improvement (e.g. option to include further openings, to add further partitions). |
| Social/ economical Suitability | - Locally available material, utilising familiar techniques - Options for further improvement - Accessibility | Local procurement, where availability and available quality permits, should be prioritised; this stimulates local economy and reduces unnecessary transportation costs. Use of well-known materials and techniques will promote the participation of the beneficiaries in construction process and its maintenance. Use of familiar construction techniques will allow families to make improvements as money becomes available. Shelters should provide options for access of disabled people. |
| Cultural suitability | - Typology according to household activities, privacy and gender as well as options/capacities of reconstruction. | Design shelters to meet local household activities, as well as local cultural requirements. The design of the shelter should enable flexible use of both available interior and exterior space. Respect design and techniques adopted by beneficiaries when building their own shelter. |
| Resource effectiveness | - Use salvaged materials. - Allow future reuse of materials. - Minimize impact on natural resources | The use of salvaged materials is encouraged when in good condition (bricks, door/window-frames, roof beams etc.). Provide best practice guidance on material selection and re-use to prevent detrimental construction methods. Select quality construction materials for transitional shelters that can further permanent solutions. Consider construction techniques that enable dismantling and reuse of materials. The choice of materials should avoid increased pressure on limited locally available natural resources. |
| Appropriate Location | - Location - Land tenure | Shelter should be constructed at or near the existing homestead, without inhibiting permanent housing process. Minimise exposure to hazards: avoid hazardous locations and apply DDR recommendations. Take account of access to livelihoods- the ability for small business and trade in or near the location. Ensure proper land rights for minimum 10-years tenure for permanent sites. |
| Site Risk Mitigation | - Cyclone - Hurricane - Earthquake - Rains and Floods | Shelter design must include earthquake and cyclone, hurricane resistant techniques (Build Back Safer). Shelters to be built on safe portions of land. Drainage of the area around the shelter to be examined. When necessary, construct water diverting features or rainwater containment. |

# 4. SHELTER RESPONSE OPTIONS AND TECHNICAL STANDARDS

4.1 Tarpaulins and plastic sheeting:

A guide to the specification and use of plastic sheeting in humanitarian relief can be downloaded from: https://www.sheltercluster.org/References/Pages/ShelterSpecifications.aspx

| Item | Recommended Minimum Standards | Notes |
| --- | --- | --- |
| Material specifications can be found from | https://www.sheltercluster.org/References/Documents/plastic_sheeting_comparison.pdf |  |
| Recommended size of individual tarpaulin or sheet | 6m by 4m. | For rolls or large area sheets it’s important that they are cut to ensure they are usable, but also maximise their number. |
| Number of tarpaulins or sheets distributed per family | Family size less than 6 members two tarpaulin or equivalent sheets per family | 2 tarpaulins is the accepted international standard. |
|  | Family size greater than 6 members a minimum of two tarpaulins or equivalent sheets per family | Additional tarpaulins should be allocated if supply permits. |

Some reference for good practice details for fixing tarpaulins and materials can be found on:

- Fixing plastic sheet to wood and rope - https://www.sheltercluster.org/References/Documents/Fixing%20Plastic%20Sheeting.pdf

- Temporary structures in hot climates-

https://www.sheltercluster.org/References/Documents/Temporary%20structures%20in%20hot%20climate.pdf

- IFRC shelter kit guidelines-

https://www.sheltercluster.org/References/Documents/IFRC%20shelter%20kit%20guidelines.pdf

- IFRC shelter kit instructions –

https://www.sheltercluster.org/References/Documents/IFRC%20shelter%20kit%20flyer%20instruction%201.pdf

- IFRC shelter kit specification  -

https://www.sheltercluster.org/References/Documents/IFRC%20Shelter%20Kit.pdf

4.2 Tents:

A guide to the use and logistics of family tents in humanitarian relief, published by UNOCHA can be downloaded from http://www.josephashmore.org/publications/tents.pdf.

| Item | Recommended Minimum Standards | Notes |
| --- | --- | --- |
| Tent materials | Equivalent to IFRC or UNHCR standard material specifications http://procurement.ifrc.org/catalogue | The family tent and the frame tent |
| Life span | Maximum recommended time for tents as a place of main habitation is 3 months. |  |
| Tent planning | Tent site planning and layout can be found on: https://www.sheltercluster.org/References/Pages/ShelterSpecifications.aspx |  |
| Fire safety | Tent ‘Fire Safety and Winterisation Leaflet’ can be found on: https://www.sheltercluster.org/References/Pages/ShelterSpecifications.aspx |  |

4.3 Shelter Kits:

Contain construction material and tools, and target those at or returning to damaged or destroyed houses in place of origin. The kit should provide necessary support to ensure that minimum sphere standards can be reached. See also: https://www.sheltercluster.org/References/Documents/IFRC%20shelter%20kit%20guidelines.pdf

| Phase and options | Material Options | Value |
| --- | --- | --- |
| Emergency Activity |  |  |
| Emergency shelter kits |  |  |
| Quantities to be provided by agencies Materials, cash/voucher (if markets are functioning) | Tarpaulins |  |
|  | Rope – nylon 12mm dia |  |
|  | Tying wire – 1.5mm dia |  |
|  | Nails – for roofing, timber |  |
|  | Tools – saw, claw hammer, shovel, pliers, etc |  |
|  | Other |  |
|  | Training, technical guidance, monitoring |  |
| Recovery Activity |  |  |
| Partially damaged house repair kit |  |  |
| Quantities to be provided by agencies Materials, cash/vouchers (if markets are functioning or alternatives like fairs can be arranged) | CGI sheets – 26ga |  |
|  | Plywood -18mm thick |  |
|  | Timber - various sizes, certified timber if possible |  |
|  | Nails – for roofing, timber |  |
|  | Tools – saw, claw hammer, shovel, pliers, etc |  |
|  | Other |  |
|  | Training, technical guidance, monitoring |  |
| Significantly damaged house repair kit |  |  |
| Quantities to be provided by agencies Materials, cash/vouchers (if markets are functioning or alternatives like fairs can be arranged) | CGI sheets – 26ga |  |
|  | Plywood – 1/4 (5mm) |  |
|  | Timber - various sizes, certified timber if possible if possible |  |
|  | Concrete materials (10mm RSB, cement, sand, gravel, CHB). |  |
|  | Nails – for roofing, timber |  |
|  | Tools – saw, claw hammer, shovel, pliers, etc |  |
|  | Other |  |
|  | Training, technical guidance, monitoring |  |
| Destroyed or severely damaged house |  |  |
| Core House | Under the ‘Rehab Housing Policy’ the GoF aims to provide these for qualifying applicants. Rural Housing to provide details | FJ$ 14,500 (figure quoted by GoF, Western Division) |
|  | Training, technical guidance, monitoring |  |
| Transitional Shelter | 21 square metres CGI covered transitional shelter with tarpaulin walls. Habitat for Humanity Fiji to provide details | FJ$ 3,000 (materials only) |
|  | Training, technical guidance, monitoring |  |

For information on selecting NFIs see: https://www.sheltercluster.org/References/Documents/Selecting%20NFIs%20for%20Shelter%202009.pdf

4.4 Temporary/Transitional Shelters:

Transitional shelters are shelter provided during the period between a disaster and the achievement of a long-term shelter solution. It provides a habitable covered living space, a secure, healthy living environment with privacy and dignity for those living within it.

The shelters are designed so that material are re-useable for when families can move onto a more permanent site, contributing towards construction of semi-permanent and permanent houses.

All Temporary/Transitional Shelters should have appropriate WASH facilities, particularly in relocation situations.

Detailed designed are to be provided by Habitat for Humanity Fiji. The following are the basic standard:

| Indicators | Standards | Foreseen Challenges |
| --- | --- | --- |
| Size | Minimum of 18m2 covered living space for a family of 5 |  |
| Timeframe | Structure needs to able to last a minimum of 12-18 months | Ability to move onto permanent site |
| Location | - Location on plot shall allow further incremental development of shelter - Allow space for DRR measures - 100 families per hectare maximum | Lack of space to add to shelter Lack of access to funds or skill to build |
| Use of salvaged material | - Only qualified salvaged materials (e.g. avoid burnt, decayed, swollen material) - Check amount of salvaged material available to beneficiaries - Design of shelters not fully to rely on availability of this type of material. | Ensure quality of salvaged materials |
| Plot preparation | - Properly clear site from physical hazards (e.g. flood and other debris, trees likely to fall, salvaged material, also from neighbouring plots) - Properly prepare site following DRR principles (good compaction of construction site) – Be aware of river silt deposit, not a quality base soil |  |
| Construction process | - If possible apply traditional, well-know construction methods based on existing skills of available labor Construction process to be speedy (pre-fabrication of components reduces on site cutting or drilling) and simplified to enable the participation of semi or un skilled labour. |  |
| Foundation | - Excavation should be deep enough to reach stable or hard soil type. - Ensure good compaction of earth - When possible ensure PCC layer beneath foundation – Raise plinth 6-9” above flood water level. See Flood risk map (attached) |  |
| Floor level | - Raise floor level to prevent ingress of low surface water - height according to location, min. 10cm |  |
| Structure Structure (cont.) | - Inform on simple solutions to improve the shelters resistance: e.g. braces, improved joists, ratio length: width, slope and overhang of roof etc.) - Ensure that frame material (eg. Wood or metal) bear the load rather than fasteners/fixings. - Current local practice to treat bamboo and wood members uses burnt engine oil or paint. - Ensure water drainage from the roofs. - Transitional shelter: Lightweight frame anchored to ground temporarily - The structures of transitional shelter should be demountable to allow the reinstallation of the shelter in a new (or original) location or the reuse of the materials. |  |
| Head height | - Flat roofs height should be 9ft (2.75m) - Double pitch roofs: -60% of shelter should have min. height of 7ft (2.1m) |  |
| Hazard | Standards | Foreseen Challenges |
| Heavy Rains and Floods | - Pitch Roofs: slope min 0,5% gradient. - Recommended extension of eaves: min. 6”. - Raise plinth level high enough to protect the base of the wall. - For block construction use plaster on external walls to increase life span of wall. |  |
| Earthquake | - Match design of shelter to local seismic risk. - Seismic resistance techniques to be incorporated into site selection, shelter form, the location of openings, foundations, bracing and ring beam connections - Openings weaken the structural integrity of walls – ensure load above the openings is transferred to other structural components. - Roof beam to overhang min. 6” on each side - Walls to integrate braced structure |  |
| Fire Hazards | - Perform site planning and disseminate information on appropriate safe use of fire near the shelter. |  |
| Cyclone, Typhoon, Hurricane/ Strong Winds | - Form of shelter: rectangular or square type (ratio length to width approx. 1:1 or 1:1.5) - Secure shelter to the ground (strong foundations, lightweight frame anchored to ground) - Roof structure with adequate strength for proposed roofing material - Apply metal strapping to reinforce roof structure to withstand hurricanes, earthquakes - Sufficient pitch to withstand winds: 2-pitched roof: min. 30°- 45°, 1-pitched roof: 12°-14° |  |

4.5 Support to Host Families:

Guidance on Host Family support:

https://www.sheltercluster.org/MENA/Syria/Pages/Technical-Guidance.aspx

4.6 Permanent Housing:

| Destroyed or severely damaged house |  |  |
| --- | --- | --- |
| Core House | Under the ‘Rehab Housing Policy’ the GoF aims to provide these for qualifying applicants. Rural Housing to provide details | FJ$ 14,500 (figure quoted by GoF, Western Division) |
|  | Training, technical guidance, monitoring |  |

# 5. CROSS CUTTIMG CONSIDERATIONS

## 5.1 Environment

https://www.sheltercluster.org/References/Pages/Environment.aspx

## 5.2 Gender

https://www.sheltercluster.org/References/Pages/Gender.aspx

## 5.3 Age

https://www.sheltercluster.org/References/Pages/Age.aspx

## 5.4 HIV/AIDS

https://www.sheltercluster.org/References/Pages/HIVAIDS.aspx

## 5.5 Disabilities

## https://www.sheltercluster.org/References/Pages/Disabilities.aspx

## 5.6 Human Rights

https://www.sheltercluster.org/References/Pages/Human-Rights.aspx

## 5.7 Livestock

https://www.sheltercluster.org/References/Pages/Livestock.aspx

# 6. INTER CLUSTER CONSIDERATIONS – to be completed

Inter cluster guidance can be found on the www.sheltercluster.org website:

https://www.sheltercluster.org/References/Pages/InterCluster-Guidance.aspx

https://www.sheltercluster.org/References/Pages/IASCGuidance.aspx

## 6.1 CCCM

N/A

## 6.2 Protection

## 6.3 WASH

## 6.4 Early Recovery

N/A

# 7. IMPLEMENTATION METHODOLOGIES

## 7.1 Cash Grants & Voucher

https://www.sheltercluster.org/References/Pages/Shelter-and-Cash.aspx

## 7.2 Cash for Work

https://www.sheltercluster.org/References/Pages/Shelter-and-Cash.aspx

## 7.3 Participation

https://www.sheltercluster.org/References/Pages/Shelter-Programming.aspx

## 7.4 Owner Driven Approach

https://www.sheltercluster.org/References/Pages/Shelter-Programming.aspx

## 7.5 Technical guidance, supervision and monitoring

Camp Planning, Collective Centres, Land Tenure, Site Selection and Urban Planning documents:

https://www.sheltercluster.org/References/Pages/SettlementPlanning.aspx

Design and Material Specification documents:

https://www.sheltercluster.org/References/Pages/ShelterSpecifications.aspx

Assessment, Monitoring & Evaluation documents:

https://www.sheltercluster.org/References/Pages/Shelter-Programming.aspx

Annexes