| Location | Puntland | Rural/Urban setting | Urban setting |
| --- | --- | --- | --- |
| Type of shelter | Transitional shelter | Material source | Imported, locally procured |
| GPS Longitude | 49°12'39.26"E | Main structural materials | Timber 2”x2”, 9mm Plywood and iron sheets |
| GPS Latitude | 11°16'14.58"N | Main cladding materials | 9mm thick Plywood, iron sheets |
| Year of construction | 2015 | Hazard resistance (earthquake) | N/A |
| Anticipated lifespan | 4-5 years | Hazard resistance (wind) | Resistant |
| Climate type | Hot weather | Hazard resistance (floods) | Good resistant |
| Disaster |  | Hazard resistance (fire) | Resistant partially |
| Time to build (man-days) | 3 days, max | Pictures uploaded | Not yet |
| Number of rooms | 1 room | BOQ uploaded | Not yet |
| Labor method | Owner-driven approach | Drawings uploaded | Not yet |
|  |  | Satellite pictures uploaded | Not yet |

SHELTER TYPOLOGIES - Somalia

Pictures of prototype

| Size of built area (m2) per unit | 16.15 sq. | Can the units be accessed by persons with disabilities without help from others? (universal design) Y/N | Yes |
| --- | --- | --- | --- |
| Cost per unit (approximately in US $) | 850-950 USD | Do the construction principles, tools and materials need specialist support during construction? | No |
| Cost of building material per unit (approximately in US $) | 700/800 USD | Does the maintenance can be done by beneficiaries themselves? (Knowledge, tools, affordable materials…) | Yes |
| Cost of labor per unit (approximately in US $) | 150 USD | Are the shelters natural disasters resistant? Y/N | Depends the nature of the disaster |
| Are the beneficiaries satisfied with their opportunity to contribute to the design/planning of the project? | Yes |  |  |

Shelter description

| Shelter description | Transitional New hybrid shelter, with a strong solid foundation and an upper structure of plywood (see below) |
| --- | --- |
| Durability and lifespan | - The foundation (1m high) is well plastered and strong to stay long - Upper wall is 9mm thick plywood which is temporary but can stay some years and easily upgradable - One weakness can be resistance of the plywood against rain, but Puntland gains less rain once a year, and especially once in 2-3 years Bossaso. - As a contribution and to enhance the ownership aspect, beneficiaries can upgrade by using oil paint/water proof to increase the lifespan |
| Live-ability/Comfort | - Cooler comp - 16.15 sq. shelter space with 70 sq. plot - 2 opposite windows for cross ventilation - Culturally acceptable |
| Notes on possible upgrades | - Flooring - Painting - Wall can be transformed into block wall - Extension can be added - Availability of materials for upgrading in the local market |
| Ecology | - All materials except sand is imported thus no effects on the environment. - Has iron sheeting roof preventing rain - Block/plywood wall preventing heat |
| Protection issues | - 7 years land tenure agreement - Good quality of Structural materials preventing theft - Has Lockable doors |

Extra Documents

SWOT analysis (Strengths, Weaknesses, Opportunities, Treats)

| Strengths | Weaknesses | Opportunities | Threats |
| --- | --- | --- | --- |
| New type Quick construction | Material imported Too permanent to be T-shelter and too temporary to be permanent | Funding | Insecurity Lack funding |

Pros and cons of the shelter

| Benefits | Limitations |
| --- | --- |
| Rectangular – can be divided Concrete base to prevent fire Good typology for the hot environment Windows for ventilation Quick construction Flood resistant Good foundation Site planning exists | Too permanent to be transitional shelter, too temporary to be permanent shelter Plaster foundation makes difficult to expand If blocks of foundation stuck out, it could be more easily expanded Plywood degradation possible Not cost effective 3 men days – questionable Size of door small Roof may need to be better anchored Provide text/description on topology Slope? |