SHELTER TYPOLOGIES – Somalia

| Location | Mogadishu / Banadir | Rural/Urban setting | Urban and rural |
| --- | --- | --- | --- |
| Type of shelter | Hybrid shelter | Material source | Locally available |
| GPS Longitude |  | Main structural materials | Timber. Use of CGI helps stiffen the structure. |
| GPS Latitude |  | Main cladding materials | CGI and plastic sheeting |
| Year of construction | 2015 | Hazard resistance (earthquake) | Limited |
| Anticipated lifespan | More than 3 years (However, the plastic sheet needs to be replaced every year if the wall materials is not upgraded | Hazard resistance (wind) | Limited wind resistance. May require bracing. |
| Climate type | Semi-arid, humid and temperate. | Hazard resistance (floods) | No, but consideration should be made to place the shelter on higher ground. |
| Disaster | Displacement caused by conflict, fire. | Hazard resistance (fire) | The timber and plastic sheeting are highly flammable. |
| Time to build (man-days) | Less than a day. Depending on the size of teams, several shelters can be erected in a day | Pictures uploaded | Yes |
| Number of rooms | 1 room | BOQ uploaded | Yes |
| Labor method | Skilled and unskilled | Drawings uploaded | Yes |
|  |  | Satellite pictures uploaded | No |

Picture of prototype

| Size of built area (m2) per unit | 16 Sq. | Can the units be accessed by persons with disabilities without help from others? (universal design) Y/N | Yes. If on raised ground, a ramp may need to be constructed. |
| --- | --- | --- | --- |
| Cost per unit (approximately in US $) | 300 USD | Do the construction principles, tools and materials need specialist support during construction? | Yes but limited especially for site planning and initial shelter. |
| Cost of building material per unit (approximately in US $) | 250 USD | Does the maintenance can be done by beneficiaries themselves? (Knowledge, tools, affordable materials…) | Construction and maintenance can be undertaken by beneficiaries, with limited training / support. |
| Cost of labor per unit (approximately in US $) | 50 USD | Are the shelters natural disasters resistant? Y/N | Offers minimum resistance to strong wind loads, floods and earthquakes. |
| Are the beneficiaries satisfied with their opportunity to contribute to the design/planning of the project | Not fully, at some point the beneficiaries have complained about the hybrid shelter because of security, the plastic materials can be torn easily. |  |  |

Shelter description

| Shelter description | An upgradable, transitional shelter made of a timber structure, corrugated galvanized iron sheet roofing and plastic sheeting wall cladding. |
| --- | --- |
| Durability and lifespan | Structure can last for more than 3 years, if protected from termite attack while the roof could last more than 5 years if humidity levels are low. However, the plastic sheeting needs to be replaced every year if the plastic sheeting is not changed. |
| Live-ability/Comfort | Covered space meets the sphere standard for a family of 4. Lacks privacy, and physical protection as the plastic sheeting can be easily cut and accessed. Not well ventilated, making the internal thermal climate quite uncomfortable. |
| Notes on possible upgrades | Wall needs to be upgraded, from plastic sheeting to another material that can offer better physical protection and privacy. Possible upgrade materials include bricks, CGI, plywood, etc. |
| Ecology | With nearly all materials (timber, plastic sheeting and nails) imported from outside the country, the local environment is largely protected from harmful exploitation. |
| Protection issues | Plastic sheeting affords limited privacy and protection against intrusions, hence the need for upgrades. |

Extra Documents

SWOT analysis (Strengths, Weaknesses, Opportunities, Treats)

| Strengths | Weaknesses | Opportunities | Threats |
| --- | --- | --- | --- |
| -Local integration -Site planning exists | -Insecure | -Land tenure | -Cyclone winds and excessive floods |

Pros and cons of the shelter

| Benefits | Limitations |
| --- | --- |
| -Cheap -Moveable and dismountable -Quick to build -Environmentally-friendly -Local markets supported -Can be upgraded -Good structure | -Insecure -Short lifespan -Fire-resistant? The timber and plastic sheeting are highly flammable. -Very hot -No privacy -Toilets? -Community participation? -Contractor-driven |