HURRICANE DORIAN REPAIRS

ISLAND OF GRAND BAHAMA

INSPECTION SHEET

NAME:				 	   PERMIT #:		     LOCATION:                                      TEL:

- Please note that there can be no alterations/conversions made to the ORIGINAL BUILDING PERMIT.

- ALL mandatory inspections are performed by the MINISTRY OF PUBLIC WORKS & THE MINISTRY OF ENVIRONMENTAL HEALTH whereas applicable.

- Failure to have the Bahamas Building Code mandatory inspections performed will prevent the Ministry of Public Works issuing a Certificate of Occupancy and therefore the structure will NOT be energized.

Please note inserts as related to the ALL building inspections:

EXTRACT FROM BAHAMAS BUILDING CODE

SECTION 312 MANDATORY INSPECTIONS

The Building Control Officer upon 48 hours’ notice exclusive of Saturdays, Sundays and declared Public Holiday from the builder shall make the following inspections and shall either approve the portion of the works completed or and shall notify the builder in writing where such work does not meet with his approval.

312.1 FOUNDATION INSPECTION shall be made when the necessary excavations have been completed forms erected and the reinforcing steel placed in position.

312.2 PILING INSPECTIONS shall be made during the driving of the piles and after all piles are driven and forms erected and reinforcing steel in place and before any concrete is poured.  The builders shall be responsible for keeping of a log of the pile driving and shall supply a copy to the Buildings Control Officer.

312.3 TIE COLUMN INSPECTION shall be made when the wall blocks have been laid, the tie columns formed and the reinforcing steel in place, but before the concrete is poured.

312.4 STRUCTURAL COLUMN INSPECTION shall be made before any blocks are laid; the columns shall be formed up, the reinforcing steel in place, but before the concrete is poured.

312.5 TIE BEAM INSPECTION shall be made when block walls are completed to tie beam level, the tie columns formed and poured, reinforcing steel in place but before the tie beam concrete is poured.

NOTE: For buildings of more than one floor and of bearing wall construction, inspections 312.3 and 312.5 shall be made at each floor.

312.6 STRUCTURAL BEAM AND SLAB INSPECTION shall be made when the beams and slabs have been formed up, reinforcing steel in place, but before the concrete is poured.

312.7 STRUCTURAL FRAMEWORK INSPECTION if the structure is of steel or other similar materials forming a frame, inspection shall take on the complete frame for single floor buildings, for larger buildings it shall be by floors before the framework is encased.  This inspection or inspections shall include any lightweight floor beam systems.

NOTE: For buildings of more than one floor of structure frame design, inspections 312.4, 312.6 and 312.7 shall be made in each floor level.

312.8 ROOF INSPECTION shall be made when the roof frame is in place; the upper surface covered with roof material, but before the ceiling is in place.

312.9 PLUMBING INSPECTIONS: The following Plumbing inspections shall be made in all buildings and at all floor levels, if the building has more than one floor.

- Roughing-in

- Plumbing in walls and baths in place for hydrostatic test

- Fixtures in place

312.10 SPECIAL INSPECTIONS shall be made of all mechanical installations immediately on completion of the work or at such intervals during the process of the works as the Buildings Control Officer or this Code may deem necessary.

312.11 OTHER INSPECTIONS shall be made if the Building Control Officer deems then necessary.

312.12 ELECTRICAL INSPECTION shall be made at the stages prescribed by the Minister charged with the responsibility for electricity.

I.E   (a)		CONDUIT ROUGHING-IN

(b)		CONDUCTORS INSTALLED AND JOINTS MADE

(c)		FINAL

THE MINISTRY OF PUBLIC WORKS SOLICITS YOUR COPERATION IN THIS MOST IMPORTANT VENTURE.

Mr. Clint Bain - Sr. Structural Engineer