Syria:  Ministry of Local Administration (MOLA)

update: 5 December 2012

Guidelines on Response to Transitional ‘Collective’ Shelters

Minimum standards of rehabilitation and essential criteria

for the rehabilitation of existing buildings as short-term collective shelters hosting internally displaced families.

Overview / Purpose

The aim of this note is to articulate the minimum and expectations for standards for the rapid rehabilitation of collective shelters in Syria.  This includes the provision of shared resources as well as accounting for the privacy, dignity, safety and social practices of the displaced persons and families in the shelters.

Criteria are proposed as guidelines based on international standards, and also local information from site visits to collective centers in various Syrian governorates.  Also accounted for is data obtained from many Initial Rapid Assessments by MoLA on relocation/resettlement and socio-cultural issues of displaced persons and families in the Syrian society due to the current crisis.

- Living Space:

- Area per individual / family:

As a minimum 12 sq.m per family is required – indicatively 3.5-4.5 sq.m per person.  Assuming the Syrian family consists of an average of 5 persons the living space is acceptable if ranging between 12 and 15 sq.m for one family.  However, this is in case space is to be divided and allocated to families.  Where families get separate shelter units, such as in Talaee camps, the privacy of the family would outweigh the surface area of the shelter. In the case of schools, a standard classroom (35 sq m) could be divided into two spaces.

2. Separations:

Light wooden frames with panels of hardboard or Gypsum board partitions are recommended as convenient for partitioning.  Lined on both sides.  Finished as appropriate.

3. Water, Sanitation, and Hygiene (WASH)

The estimation of (Male/Female) ratio is imperative for deciding allocation of spate facilities, and WASH solutions must respect the cultural norms of the society and the relevant Hygiene standards.  The following minimum standards are recommended.

- Drinking water

Fresh water for drinking and cooking purpose should meet the following the minimum criteria:

Minimum International Standards for clean water are as follows;

- Free from faecal coliforms

- Turbidity < 5NTU (Nephelometric Turbidity Units)

- With disinfected water, 0.5mg/ liter of free chlorine residue at point of

- Delivery (Refer to page 67 of Sphere Standards on water and Sanitation)

When there is a need to provide the displaced families with fresh drinking water separately, the calculation of fresh water requirement of the number of families that would use the tank should be based on 3 liters per person per day (considering multipurpose use of fresh water in drinking, cooking, and preparing children meals etc.)

International standards on sanitation and washing facilities are 1 toilet for 20 persons, but practically 1 toilet would be convenient for a maximum of 30 persons, which means a minimum of one toilet for 6 families with a maximum distance of 50 m from the shelter to toilets.

Also, 1 shower (including hot water) for 40 persons, i.e., a minimum of 1 shower for every 8 families with a maximum distance of 50 m from showers to shelters.

The water storage tanks should be designed after calculation the water requirement of the number of families that will sue the tank.  The water requirement would be approximately 50 liters per person per day as the Syrian society generally has some special daily use of water (religious and cultural reasons), (From Lessons Learnt, 15 liters per person per day according to sphere standard is too low).

Water Heaters:

To be provided with proper electric installation as per the number of families served at each water point/showers and as feasible to on install on site.

- Sewage:

The diameter of sewage pipes should be calculated based on a minimum of 50 liters per person per day, and equivalent minimum volume of septic tank where feasible.

2-  Other Facilities:

- Electricity:

Lighting: 1 bulb, minimum 2 separately wired sockets per room.

Min. assured 2 KW per family. Cable 2.5 mm2 (family level), cable 6 mm2 (from connection to floors) depending on the size of building.

1 fuse per family.

- Kitchen facilities:

One cooking stove for 20 familes in communal facility with associated practical communal cooking equipment

- Communal clothes washing facilities:

A suitable space should be developed with water, drainage and washing equipment to serve as a communal clothes washing facility

- Doors and windows:

Wooden lockable doors when wooden partitioning is adopted. (a minimum of 1 per room, glass and wind / draft proof).

- Stairs:

Repaired and hand railed

- Flooring:

Light carpet (Moquette) for family allocated rooms – min 12 sq.m per family.

- Building outside walls and non-structural elements:

Patching as required

- NFI:

Each family is to be provided with sanitation related NFIs procured locally including a solid waste bin, a bucket, mop, broom, brush. etc. plus collective bins for a specified number of families as feasible per each site.

- Existing walls and non-structural elements:

Plastering where needed.

- Heating:

1 communal heater (gas, electric etc.) for a group of families

(e.g. 10 family) when appropriate enough shared space (e.g. social

activities hall) is applicable. 1 individual heater for 1 family (can be

adopted in some cases based on the technical condition of the shelter,

number of  special need persons who need to special care (e.g. disabled

persons, elderly persons, children and infants) in the shelter.

- Overall shelter safety:

- Minimum of 2 fire extinguishers per site (one in communal kitchen

- All fittings in consideration of children and safety

- Social services to be linked in to shelter sites (child  friendly spaces and activities0

- All shelter rehabilitation works to be done in close coordination with Shelter management / supervision for ongoing activities – maintenance (sanitation) and recurrent costs

- Authorizations to work at shelter sites:

- MOLA to provide and ensure all legal aspects covered

***