	
	BILL OF QUANTITIES – FULL SHELTER KIT
	
	

	Item 

Number
	Description of Materials
	Quantity
	UOM

	
	
	
	

	1
	Cement, Portland, 40kg/bag
	3
	bag

	2
	Sand, fine
	0.5
	cu.m.

	3
	Gravel, ¾’’, washed
	0.5
	cu.m.

	4
	Steel re. bars, 10mm dia. X 6m, deformed, mild steel
	5
	pcs.

	5
	Steel re. bars, 8mm dia. X 6m, deformed, mild steel
	5
	pcs.

	6
	G.I. Roofing Sheet, corrugated, 26ga. (0.4mm thick) X 10 ‘
	18
	sheets

	7
	Ridge roll, G.I. plain, 26ga (0.4mm thick) X 8’
	3
	pcs.

	8
	Tie wire, galvanised, No. 16
	0.5
	kg.

	9
	Umbrella nail, twisted, 2 ½ ‘’
	3.5
	kg.

	10
	Nail 4’’
	4.5
	kg.

	11
	Nail 3’’
	1
	kg.

	12
	Nail 2 ½ ‘’
	3
	kg.

	13
	Nail 2’’
	0.5
	kg.

	14
	Hinge, 2’’ X 4’’, square corner, brass (2 pcs per set)
	7
	sets

	15
	Vulca seal, 220 grams, easy squeeze pack
	1
	pack

	16
	Coco Lumber 6’’ X 6’’ X 10’ (6pcs)
	180
	bd.ft.

	17
	Coco Lumber 2’’ X 5’’ X 14’ (3pcs)
	35
	bd.ft.

	18
	Coco Lumber 2’’ X 4’’ X 10’ (15pcs)
	100
	bd.ft.

	19
	Coco Lumber 2’’ X 3’’ X 10’ (50pcs)
	250
	bd.ft.

	20
	Coco Lumber 2’’ X 2’’ X 10’ (52pcs)
	173.3
	bd.ft.

	21
	Bamboo, split, 2’’ X 10’
	60
	pcs.

	22
	Bamboo Mat (Sawali) circa. 2.5 X 4 m
	6
	pcs.


Coco lumber minimum specifications:
A. For Timber 6’’ X 6’’ X 10’:

· Rough and straight
· Sawn or cut from the lower part of the coconut tree

· The timber must be from a coco tree at least 10 years old

· Maximum curve tolerance = 3’’

B. For all other timber sizes:

· Rough and straight

· The timber must be from a coco tree at least 10 years old

· Maximum curve tolerance = 5’’

Recommendations:

· It is highly recommended to use Portland cement for the foundation footings due to its’ higher rate of strength development and long term durability compared to other types of cement available on the market.

· Vulca seal is likewise recommended for the roof sealant as experience shows that it is more durable, economic and water resistant.

· The size of the bamboo mat (sawali) varies from place to place, because it is produced locally. The size of this material should be negotiated with the suppliers during the quotation period to produce the size required above.

