	
	BILL OF QUANTITIES – ROOF KIT
	
	

	Item 

Number
	Description of Materials
	Quantity
	UOM

	
	
	
	

	1
	G.I. Roofing Sheet, corrugated, 26ga. (0.4mm thick) X 10 ‘
	18
	sheets

	2
	Ridge roll, G.I. plain, 26ga (0.4mm thick) X 8’
	3
	pcs.

	3
	Umbrella nail, twisted, 2 ½ ‘’
	3.5
	kg.

	4
	Nail 4’’
	3
	kg.

	5
	Nail 3’’
	1
	kg.

	6
	Vulca seal, 220 grams, easy squeeze pack
	1
	pack

	7
	Coco Lumber 2’’ X 5’’ X 14’ (3pcs)
	35
	bd.ft.

	8
	Coco Lumber 2’’ X 4’’ X 10’ (15pcs)
	100
	bd.ft.

	9
	Coco Lumber 2’’ X 3’’ X 10’ (24pcs)
	120
	bd.ft.

	10
	Coco Lumber 2’’ X 2’’ X 10’ (7pcs)
	23.3
	bd.ft.


Coco lumber minimum specifications:
For all timber sizes:

· Rough and straight

· The timber must be from a coco tree at least 10 years old

· Maximum curve tolerance = 5’’

Recommendations:

· Vulca seal is recommended for the roof sealant as experience shows that it is more durable, economic and water resistant.

