Shelter NFI Common Standard Guidance for Flood Response 2015

endorsed by Emergency Shelter  and Camp Management Cluster Malawi

TARPAULIN – 4m x 6m Quantity per kit: 2

Tarpaulins should be made of woven plastic. They are the key component of the shelter kit. They can be used for roofing, walls and floor covering.

SHELTER TOOL KIT – Quantity per kit: 1 Kit Contains:

- ROPE – Quantity per kit: 1x30 m

A universal fixing with many uses, such as binding timbers together, stabilising structures or fixing tarpaulin in place.

- HANDSAW – Quantity per kit: 1

The handsaw is for cutting timber to required size. Do not use on metal or other hard materials.

- NAIL for roof sheets – Quantity per kit: ½ Kg (approximately 1lb.)

Nails for roofing sheets are have a domed head and a washer. They are used for fixing tarpaulins to timber, and for fixing corrugated iron sheet. When used properly the nails will not let rain seep through their own holes.

- SHOVEL – Quantity per kit: 1

The shovel is useful when preparing the foundation of a shelter. It can also be used for digging trenches around the shelter if is raining. It can be used as an everyday tool when disaster affected people re-establish their livelihoods.

- HOE – Quantity per kit: 1

The hoe can be used to prepare the ground for a shelter. It can be used as an everyday tool when disaster affected people re-establish their livelihoods.

- MACHETE – Quantity per kit: 1

The machete is shaped like a very big knife. It is  also useful for clearing sites, shaping timber and cutting extra materials.

- SHEARS – Quantity per kit: 1

The shears are for cutting metal roofing sheets, and wire. Cutting wire or nails with the shears will damage them.

- NAILS, LARGE – Quantity per kit: ½ kg (approximately 1lb.)

These are 75mm long, simple nails for building with timber.

- NAILS, SMALL – Quantity per kit: ½ kg (approximately 1lb.)

These are 40mm long, simple nails for building with timber.

- TIE WIRE – Quantity per kit: 25m (approximately 82 ft.)

1.5 mm diameter tie wire can be used for binding timber or for fixing tarpaulins. It can also be used for reinforcing structures and for diagonal bracing.

- CLAW HAMMER – Quantity 1

For driving and removing nails, and for working with other tools to make joints.

- WOVEN SACK – Quantity 1

Woven sack is for easy packaging and transportation of the shelter kit

Supplementary Wood: - 4 wooden beams 4 m long and section of 75mm x 75mm or 3" x 3"

The Shelter Tool kit can only be applied if supplemented with wood sticks, poles or planks.

If sticks are available locally they can be used in emergency situation, mindful of environmental concerns related to depleting the remaining natural forest resources in the country, the already done damage by the floods and the potential for community tensions between displaced and host communities the Emergency Shelter Custer is suggesting that agencies should provide along with tarpaulins and shelter tool kits outlined above:

In Malawi 4 x 2 – app 6m long beams are most commonly available in the market. Therefore 3 such beams should be provided along with the tarpaulins and shelter tool kits mentioned above.

Agencies should source wood supplies only through sustainable forest resource supplies, readily available in the country.

Training:

Communities will need support through staff/volunteers, trained on the use of shelter kits and materials provided by agencies. Materials should not just be handed out without prior or parallel training to affected population.