Some important considerations of CGI Sheeting 
	ADVANTAGES
	DISADVANTAGES

	• Durable if properly installed 
	• Requires more skills to fix and flash properly

	• Reduces prevalence of insects, rodents, snakes in the ceiling, except where a “false” or dropped ceiling is used 
	• The house becomes very hot – poor insulating properties

	• Reduces the incidence of moulds and fungi 
	• Extremely noisy during rainfall

	• Weather resistant if properly installed
	• Reduces air flow

	• Decreases fire risk
	• Requires larger and straighter timbers for nailing the CGI securely, in turn requires stronger wall fixings

	• Improves hygiene
	• Requires nails and flashings

	
	• Requires more tools to construct

	
	• Improper nailing damages the CGI sheets and causes leakages

	
	• Expensive to purchase and transport

	
	• When damaged it is harder to patch or repair

	
	


Notes:

Oxidation and leaching is very fast in coastal regions with salinity in atmosphere.

Anticorrosive coatings and weather proof coatings recommended only after first oxidation to reduce flaking of coats due to weathering.

Installation in heavy wind and cyclonic zones require proper wind ties and hooks with the roofing members to prevent from loosening of sheets due to wind flow.
