| Acute Emergency Shelter Kit – Technical Specifications |  |
| --- | --- |
| Airliftable acute kit, immediate response Volume: 0.0468 m3 Weight: 15 kg Indicative material cost: USD 47.50 Road transportation cost: USD 25 Air transportation cost: USD 45.4 References for design: - ‘the 2 poles version’, - ‘the tree version’, - ‘the clothesline’ | Composition: - 2 Plastic Sheeting 4x5m, 190g/m² minimum, banded and eyelet preferable. - Nylon rope 30 meters, diameter 6 or 7 mm - 4 to 6 wooden pegs (replacement of top bended rebar 16mm dia, 30 cm length) - Medium duty hoe (1 per HH or according type of distribution); or adapted according situations. In the basic version, the kit does not reach standards and has to be adapted according location of the dispatch. |

# Design type and main specification and dimensions

- Acute Emergency Shelter, ‘minimal poles’ version

As an immediate response, this design is based on an airlift where logistics constraints are severe or where populations are extremely mobile. It does not meet standards and should be upgraded wherever possible. Where feasible, space and logistics allowing, agencies shall distribute the standard emergency shelter kit instead.

As observed on Don Bosco site (Juba) where Medair distributed acute e-shelter kits in January, beneficiaries improved their design with separate donation of bamboo and salvaging some materials. The Vault shape was done adding flexible bush branches collected in the site surrounding.

This type of shelter will definitely need a renewal before the rainy season if IDPs remain, adding extra materials to increase durability and space. The lifespan of the shelter is 3-6 months maximum, depending number of displacements faced by IDPs.

For fixing the Acute Emergency Shelter to the ground several techniques could be employed: pegging using wooden or rebar pegs, or just digging light trenches and burying the plastic sheeting weighted by few stones, then backfilling and compacting (more permanent and needs a spade).

- Acute Emergency Shelter,’ tree hung’ version

This variant uses the available elements, tying the rope to a tree trunk. Several shelters could be easily set up with one plastic sheeting on the ground and another to cover the space. Eyelet on each extremity will help to fix the plastic sheeting on the rope and further each sides of the shelter have to be properly weighted or buried in a small trench.

The distance between the fixation point on the trunk and the peg of the rope on the ground shall be between 6 to 7.5 meters maximum.

- Acute Emergency Shelter, ‘clothesline’ version

Initially proposed by DRC for the POC of Malakal, this variant of Acute Emergency Shelter uses vertical elements in order to line up several acute e-shelters. In the simplest version only enough space to sleep is provided, other activities such as cooking, bathing are not possible. This variant is recommended only in overcrowded areas and under acute emergency due to the unsustainable living conditions.