| Activity | Material Options |  | Value PHP |
| --- | --- | --- | --- |
| Emergency shelter |  |  |  |
| CASH | Unconditional CASH grant (spent on shelter items of their choice) 3 days work for 2 people |  | 5,000 |
| Quantities to be provided by agencies Materials | Tarpaulins X 2 | Per Family Woven plastic, Width 4 m Length Sheet of 6M. Weight 170g/m2 +/- 5%, plus 10% for the reinforcement, Bands under ISO 3801. |  |
|  | Rope – nylon 12mm dia | Polypropylene, E.G Black, Diam. 12mm, Twisted |  |
|  | Tying wire – 1.5mm dia | Tie Wire, Galvanised, E.G Diam. 1.5 Mm, 25m, Roll |  |
|  | Nails – roofing | For Roof Sheets, e.g galvanised + watertight washer 0.5kg |  |
|  | Nails - iron, for wood | e.g 75mm, 0.5kg |  |
|  | NAIL - iron, for wood | e.g 40mm, 0.5 kg |  |
|  | HANDSAW, for timber | e.g 400mm blade |  |
|  | Hammer | e.g 750g, wood handle |  |
| N:B- TOOLS ARE NOT FOR REPAIR OF DAMAGED HOUSING - IMPERATIVE TO ASSESS DAMAGE |  |  |  |
| Tents | FAMILY to recognised international standards and fit for Philippines climatic conditions to last for a period of 6 months |  |  |
| Transitional shelters | To be confirmed |  |  |