Excerpt from WASH cluster’s Operational Principles and Practices

dated 14 January 2012

The maximum number of users per toilet, shower, handwashing station and solid waste bin in evacuation centres and homebased areas in affected areas must not exceed:

|  | Toilet Type | Latrine Ratios | Shower Ratio | Handwashing | Solid waste bins |
| --- | --- | --- | --- | --- | --- |
| School EC | Perm Septic to meet School requirements, supplemented by portalets + use of existing facilities with regular desludging | 1:50 | 1:100 | 1:10 toilets but within 10m of toilet. | 1:10 HH |
| EC Other | Semi-Perm Septic supplemented by portalets + use of existing facilities with regular desludging | 1:50 | 1:100 | 1:10 toilets but within 10m of toilet. | 1:10 HH |
| Temporary Learning Centres | Portalets / Semi Perm WS Pit1 / Urinals + Use of School latrines with regular desludging | 1:55 students | nil | Group Handwashing Facilities min 15 taps. |  |
| Tented Transit Site Rural | Semi-Perm WS Pit1 / Portalets | 1:40 | 1:80 | 1:10 toilets but within 10m of toilet. | 1:10 HH |
| Tented Transit Site Urban | Semi-Perm Septic + use of existing facilities with regular desludging / Portalets | 1:40 | 1:80 | 1:10 toilets but within 10m of toilet. | 1:10 HH |
| Bunkhouse Transit Site Rural | Semi-Perm WS Pit1 | 1:20 | 1:40 | 1:10 toilets but within 10m of toilet. | 1:10 HH |
| Bunkhouse Transit Site Urban | Semi-Perm Septic | 1:20 | 1:40 | 1:10 toilets but within 10m of toilet. | 1:10 HH |
| Permanent Site | Family Latrines (Perm Septic) | Per Household. |  |  |  |

Note:

- DOH/CHO approval for use of WS Pit Latrines in lieu of full septic pit will be site.  A site inspection by DOH/CHO will consider groundwater depth and usage, soil type and planned design life in making this decision.

- Women to male toilets should be built at approximately 4 Female toilets  to every 3 Male Toilets

- 1 in 10 toilets should be disabled accessible, with a minimum of 1 toilet per site.