Shelter Interventions Quality Check list

| Documents Status | Version | Status | Effective date | Next revision |
| --- | --- | --- | --- | --- |
|  | 1.0 | Final | 1 April 2021 | March 2021 |

General Notes:

- This checklist is to help the engineers and field workers to check the work compliance and quality.

- This checklist is not a replacement for existing partners’ internal quality assurance and quality procedures.

- The guidance includes main elements only, the cluster partners to include all other elements based on the intervention and shelter structure type.

- Thorough instructions and explanations should be given to the contractors or the beneficiaries to comply and complete the works according to the agreed specifications.

- Engineers and filed workers to judge themselves about the maximum possibilities to apply the standards considering the security and risks in the field.

- All the materials and inputs used in the shelter structures should be approved by the partners’ inspectors and engineers.

- Close follow-up and minoring at all work phases are necessary to ensure overall quality.

- The partners to describe the specifications of the items in a clear way including weights, dimensions, technical specifications, standards, and other essential information with allowable variations in the bill of quantities and/or the contract with suppliers, contractors, and beneficiaries.

- The HH/ community satisfaction is one of the key elements in the implementation process of shelters taking into consideration the HH and community specificity and the predominant culture circumstance through applying to do no harm principle and the principle of no impose.

- The checklist mainly applicable for simple shelters like tents, prefab units, bricks or concrete with steel sheet roof,.... for concrete houses more elements should be added through the implementing partners.

| # | Item | Item description | Comply / Not comply | Comments and explanation |
| --- | --- | --- | --- | --- |
| 1 | Location | Safe location, far from flood prone, and wastewater collection pond or other risks such as settlers violence ,military orders…etc |  |  |
|  |  | The location leveled |  |  |
|  |  | The site prepared with accepted , compacted materials |  |  |
|  |  | The location and site preparation comply with the mentioned specifications in the BoQ or the contract |  |  |
| 2 | Floor | Smooth, level, anti-slip floor in all spaces (suitable for children, elderly and disabled movement) with minimum one layer protection from natural soil like concrete, tiles or other acceptable materials |  |  |
|  |  | Wall to floor connection is sealed to prevent water infiltration, insects and rodents |  |  |
|  |  | No seen variations in the floor level |  |  |
|  |  | Floor level raised from outside level at a minimum of 10 to 15 cm to prevent water infiltration or flooding |  |  |
|  |  | The floor complies with the mentioned specifications in the BoQ or the contract |  |  |
| 3 | External and Internal Walls | All the walls built from acceptable materials (bricks, stones, sandwich panel, gypsum broads …) |  |  |
|  |  | All the walls are free from cracks or holes and are finished to smooth render that prevents water ingress |  |  |
|  |  | All external walls are insulated for thermal comfort |  |  |
|  |  | At least the internal surfaces are plastered, painted or treated in accepted way |  |  |
|  |  | all wires , steel bars, and other materials removed |  |  |
|  |  | All sharp surfaces and edges are treated to prevent accident or injury |  |  |
|  |  | All external stone facades are adequately pointed and grouted. |  |  |
|  |  | Structurally all the walls are joined and fixed together, in the ground and the roof to prevent any structural failure |  |  |
|  |  | The external walls comply with the mentioned specifications in the BoQ or the contract |  |  |
| 4 | Roof | Roof is suitable to control heat. |  |  |
|  |  | Roof is sealed and there are no leaks, ensuring fixing gutters and channels for rainwater drainage in the roof. |  |  |
|  |  | Roof has adequate slope for drainage with adequate water gutter |  |  |
|  |  | No spaces between the roof and walls |  |  |
|  |  | All roof structural parts are adequate and fixed to resist wind, rain, snow and other loads |  |  |
|  |  | The roof height from the lowest point not less than 2m |  |  |
|  |  | Sandwich panel to connect all shelter main facilities together if applicable |  |  |
|  |  | The roof is insulated with proper materials to prevent leakage. |  |  |
|  |  | The roof elements comply with the mentioned specifications in the BoQ or the contract |  |  |
|  |  | ensuring a suitable, safe and adequate welding works where applicable |  |  |
| 5 | Frame/structural supports | Frame materials are securely fixed to the ground |  |  |
|  |  | Frame materials are able to support the weight of the roof and any additional live or dead load expected during different seasons like wind and snow (engineering experience) |  |  |
|  |  | The frame/structural supports comply with the mentioned specifications in the BoQ or the contract |  |  |
| 6 | External and internal Doors | Solid and lockable external door into the shelter |  |  |
|  |  | Steel doors painted with prime and oil coats. |  |  |
|  |  | The doors are fixed with rigid frame |  |  |
|  |  | The doors not make any obstacles or risks for users |  |  |
|  |  | The wooden doors frame and surfaces treated to avoid to resist water and damp |  |  |
|  |  | Doors are adequate for PWD if applicable |  |  |
|  |  | The doors comply with the mentioned specifications in the BoQ or the contract |  |  |
| 7 | Windows | Window units are sealed (from rain water and wind) |  |  |
|  |  | Ventilation into habitable rooms - living rooms, bedrooms, kitchen, bathroom and toilet of minimum 5% of floor area per room |  |  |
|  |  | Natural light into habitable rooms of minimum 10% floor area per room |  |  |
|  |  | Fixing additional external steel protection mesh where needed |  |  |
|  |  | Fixing insect net to the windows if needed |  |  |
|  |  | The windows specifications comply with the mentioned specifications in the BoQ or the contract |  |  |
| 8 | Toilets and showers | The toilet with adequate space and dimension and consider the PWDs needs if any. |  |  |
|  |  | Toilet door is solid and lockable |  |  |
|  |  | Safe sewage disposable system (septic, cesspit, connection to municipal sewage system) |  |  |
|  |  | Functioning mobile or fixed internal or external latrine per shelter |  |  |
|  |  | The toilets internal faces ceramic with acceptable quality and colors |  |  |
|  |  | the toilets and shower floor level fit with external surrounding level with at least 1cm lower to allow easy clean and avoid water flush outside |  |  |
|  |  | for mobile latrines the specifications match the common and known standards |  |  |
|  |  | Proper workable piping and mechanical connections |  |  |
| 9 | Kitchen | The kitchen with adequate space and ventilation |  |  |
|  |  | Including counter and connected to water source and drainage system. |  |  |
|  |  | Including at least lower good quality sandwich wooden cabinet, or wooden/Aluminum doors for kitchen with marble partitions |  |  |
|  |  | Proper workable piping and mechanical connections |  |  |
| 10 | Covered space | Where applicable and possible the Minimum covered space 5 m2 per person (including kitchen, corridors, bathroom or toilet facilities) |  |  |
| 11 | WASH Minimum standards | At least 1 water storage tank of 1000 L capacity and water tank base (30 cm height )with approved quality |  |  |
|  |  | Kitchen, latrine and shower taps are connected to water source with protected and covered pipes either GS or HDPE with approved quality |  |  |
|  |  | Hot water source with solar panels is installed and comply with the mentioned specifications in the BOQ or the contract |  |  |
| 12 | Electrical connections and lighting system | All electrical connections, switches and wiring are insulated and in good working conditions and connected to an earthing system |  |  |
|  |  | All the electrical wires and cables are protected accordance to agreed standards |  |  |
| 13 | Accessibility | The shelter is accessible and includes additional elements for PWDs |  |  |
|  |  | Steel railing and adequate external stairs if needed |  |  |